# STEEL POLE STRUCTURAL ANALYSIS

| Des. No. Rev. No. Client: Project: --Owner -- Pole: 10.668M Steel Pole,8 |  |  | 0 Sides,Direct | Buried,Desigh Load =500kgf |  |  | Date: |  | Apr | 17, 2026 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pole Overall Height: Pole Effective Heigh |  |  | 10668 mm 8991 mm | Loading: |  |  | Design Load, Pw: |  |  | 500 kgf |  |  |
| Embedment Depth: |  |  | 1677 mm |  |  |  | Application of Lo Vertical Load: |  |  | 500 mm 165 kgf |  |  |
| Type: Material: Steel Yield Strength |  |  | 355 MPa | Factors: |  |  | Vertical Load Fac |  |  | 1 |  |  |
| Allowable Stress: |  |  | 355 MPa |  |  |  | Ultimate Load, Pu |  |  | 500 |  |  |
|  |  |  |  |  |  |  | Safety Factor: |  |  | 1 |  |  |
| Pole Properties: |  |  |  |  |  |  |  |  |  |  |  |  |
| Tip Outside Diameter: Butt Outside Diameter: Structural Plate Thickness: Galvanized Weight: |  |  |  | 127 mm 248 mm 3 mm 165.46 kg |  |  | Shape: 8 Sides Taper: 11.34 mm/m |  |  |  |  |  |
| Stress Calculation: |  |  |  |  |  |  |  |  |  |  |  |  |
| Loc | Dist | Wall | Outside |  | Modulus | Section | Mom | Axial | Combined |  | Safety | No |
| Name | from Top | Thickness | Diameter | Area | Inertia | Modulus | at x | Load | Stress |  | Ratio |  |
|  | mm | mm | mm | mm2 | mm4 | mm3 | kN-m | kN | MPa |  |  |  |

260 /

SqrtFy	b/ σ	Proportionalit

| at Pw | 650 | 3.00 | 134.4 | 1308.47 | 2.98E+06 | 4.44E+04 | 0.74 | 1.69 | 17.86 | 19.88 | 1 | 35.03 | 16.06 | IN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 853 | 3.00 | 136.7 | 1331.44 | 3.14E+06 | 4.59E+04 | 4.18 | 1.71 | 92.29 | 3.85 | 2 | 35.03 | 16.38 | IN |
|  | 1057 | 3.00 | 139.0 | 1354.42 | 3.30E+06 | 4.76E+04 | 5.18 | 1.73 | 110.14 | 3.22 | 3 | 35.03 | 16.70 | IN |

y

| Loc Name | Dist from Top mm | Wall Thickness mm | Outside Diameter mm | Area mm2 | Modulus Inertia mm4 | Section Modulus mm3 | Mom at x kN-m | Axial Load kN | Combined Stress MPa | Safety Ratio | No | 260 / SqrtFy | b/ σ | Proportionalit y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1260 | 3.00 | 141.3 | 1377.39 | 3.48E+06 | 4.92E+04 | 6.17 | 1.76 | 126.74 | 2.80 | 4 | 35.03 | 17.01 | IN |
|  | 1463 | 3.00 | 143.6 | 1400.36 | 3.65E+06 | 5.09E+04 | 7.17 | 1.78 | 142.20 | 2.50 | 5 | 35.03 | 17.33 | IN |
|  | 1667 | 3.00 | 145.9 | 1423.34 | 3.83E+06 | 5.26E+04 | 8.17 | 1.80 | 156.58 | 2.27 | 6 | 35.03 | 17.65 | IN |
|  | 1870 | 3.00 | 148.2 | 1446.31 | 4.02E+06 | 5.43E+04 | 9.16 | 1.83 | 169.98 | 2.09 | 7 | 35.03 | 17.97 | IN |
|  | 2074 | 3.00 | 150.5 | 1469.28 | 4.22E+06 | 5.61E+04 | 10.16 | 1.85 | 182.46 | 1.95 | 8 | 35.03 | 18.29 | IN |
|  | 2277 | 3.00 | 152.8 | 1492.26 | 4.42E+06 | 5.79E+04 | 11.16 | 1.87 | 194.09 | 1.83 | 9 | 35.03 | 18.61 | IN |
|  | 2480 | 3.00 | 155.1 | 1515.23 | 4.63E+06 | 5.97E+04 | 12.15 | 1.90 | 204.93 | 1.73 | 10 | 35.03 | 18.92 | IN |
|  | 2684 | 3.00 | 157.4 | 1538.21 | 4.84E+06 | 6.15E+04 | 13.15 | 1.92 | 215.04 | 1.65 | 11 | 35.03 | 19.24 | IN |
|  | 2887 | 3.00 | 159.7 | 1561.18 | 5.06E+06 | 6.34E+04 | 14.15 | 1.95 | 224.45 | 1.58 | 12 | 35.03 | 19.56 | IN |
|  | 3090 | 3.00 | 162.1 | 1584.15 | 5.29E+06 | 6.53E+04 | 15.14 | 1.98 | 233.23 | 1.52 | 13 | 35.03 | 19.88 | IN |
|  | 3294 | 3.00 | 164.4 | 1607.13 | 5.52E+06 | 6.72E+04 | 16.14 | 2.00 | 241.41 | 1.47 | 14 | 35.03 | 20.20 | IN |
|  | 3497 | 3.00 | 166.7 | 1630.10 | 5.76E+06 | 6.92E+04 | 17.14 | 2.03 | 249.04 | 1.43 | 15 | 35.03 | 20.52 | IN |
|  | 3700 | 3.00 | 169.0 | 1653.07 | 6.01E+06 | 7.11E+04 | 18.13 | 2.06 | 256.15 | 1.39 | 16 | 35.03 | 20.83 | IN |
|  | 3904 | 3.00 | 171.3 | 1676.05 | 6.26E+06 | 7.31E+04 | 19.13 | 2.08 | 262.77 | 1.35 | 17 | 35.03 | 21.15 | IN |
|  | 4107 | 3.00 | 173.6 | 1699.02 | 6.52E+06 | 7.52E+04 | 20.12 | 2.11 | 268.94 | 1.32 | 18 | 35.03 | 21.47 | IN |
|  | 4310 | 3.00 | 175.9 | 1721.99 | 6.79E+06 | 7.72E+04 | 21.12 | 2.14 | 274.69 | 1.29 | 19 | 35.03 | 21.79 | IN |
|  | 4514 | 3.00 | 178.2 | 1744.97 | 7.07E+06 | 7.93E+04 | 22.12 | 2.17 | 280.03 | 1.27 | 20 | 35.03 | 22.11 | IN |
|  | 4717 | 3.00 | 180.5 | 1767.94 | 7.35E+06 | 8.15E+04 | 23.11 | 2.20 | 285.01 | 1.25 | 21 | 35.03 | 22.43 | IN |
|  | 4921 | 3.00 | 182.8 | 1790.91 | 7.64E+06 | 8.36E+04 | 24.11 | 2.23 | 289.64 | 1.23 | 22 | 35.03 | 22.74 | IN |
|  | 5124 | 3.00 | 185.1 | 1813.89 | 7.94E+06 | 8.58E+04 | 25.11 | 2.26 | 293.94 | 1.21 | 23 | 35.03 | 23.06 | IN |
|  | 5327 | 3.00 | 187.4 | 1836.86 | 8.24E+06 | 8.80E+04 | 26.10 | 2.29 | 297.93 | 1.19 | 24 | 35.03 | 23.38 | IN |
|  | 5531 | 3.00 | 189.7 | 1859.83 | 8.56E+06 | 9.02E+04 | 27.10 | 2.32 | 301.63 | 1.18 | 25 | 35.03 | 23.70 | IN |
|  | 5734 | 3.00 | 192.0 | 1882.81 | 8.88E+06 | 9.25E+04 | 28.10 | 2.35 | 305.07 | 1.16 | 26 | 35.03 | 24.02 | IN |
|  | 5937 | 3.00 | 194.3 | 1905.78 | 9.21E+06 | 9.48E+04 | 29.09 | 2.38 | 308.25 | 1.15 | 27 | 35.03 | 24.34 | IN |
|  | 6141 | 3.00 | 196.7 | 1928.75 | 9.54E+06 | 9.71E+04 | 30.09 | 2.41 | 311.19 | 1.14 | 28 | 35.03 | 24.65 | IN |
|  | 6344 | 3.00 | 199.0 | 1951.73 | 9.89E+06 | 9.94E+04 | 31.09 | 2.44 | 313.90 | 1.13 | 29 | 35.03 | 24.97 | IN |
|  | 6547 | 3.00 | 201.3 | 1974.70 | 1.02E+07 | 1.02E+05 | 32.08 | 2.48 | 316.41 | 1.12 | 30 | 35.03 | 25.29 | IN |
|  | 6751 | 3.00 | 203.6 | 1997.68 | 1.06E+07 | 1.04E+05 | 33.08 | 2.51 | 318.71 | 1.11 | 31 | 35.03 | 25.61 | IN |
|  | 6954 | 3.00 | 205.9 | 2020.65 | 1.10E+07 | 1.07E+05 | 34.08 | 2.54 | 320.83 | 1.11 | 32 | 35.03 | 25.93 | IN |

| Loc Name | Dist from Top mm | Wall Thickness mm |  | Outside Diameter mm | Area mm2 | Modulus Inertia mm4 | Section Modulus mm3 | Mom at x kN-m | Axial Load kN | Combined Stress MPa | Safety Ratio | No | 260 / SqrtFy | b/ σ | Proportionalit y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7158 | 3.00 |  | 208.2 | 2043.62 | 1.14E+07 | 1.09E+05 | 35.07 | 2.58 | 322.77 | 1.10 | 33 | 35.03 | 26.25 | IN |
|  | 7361 | 3.00 |  | 210.5 | 2066.60 | 1.17E+07 | 1.12E+05 | 36.07 | 2.61 | 324.55 | 1.09 | 34 | 35.03 | 26.56 | IN |
|  | 7564 | 3.00 |  | 212.8 | 2089.57 | 1.21E+07 | 1.14E+05 | 37.06 | 2.64 | 326.17 | 1.09 | 35 | 35.03 | 26.88 | IN |
|  | 7768 | 3.00 |  | 215.1 | 2112.54 | 1.25E+07 | 1.17E+05 | 38.06 | 2.68 | 327.64 | 1.08 | 36 | 35.03 | 27.20 | IN |
|  | 7971 | 3.00 |  | 217.4 | 2135.52 | 1.30E+07 | 1.19E+05 | 39.06 | 2.71 | 328.97 | 1.08 | 37 | 35.03 | 27.52 | IN |
|  | 8174 | 3.00 |  | 219.7 | 2158.49 | 1.34E+07 | 1.22E+05 | 40.05 | 2.75 | 330.17 | 1.08 | 38 | 35.03 | 27.84 | IN |
|  | 8378 | 3.00 |  | 222.0 | 2181.46 | 1.38E+07 | 1.24E+05 | 41.05 | 2.79 | 331.24 | 1.07 | 39 | 35.03 | 28.16 | IN |
|  | 8581 | 3.00 |  | 224.3 | 2204.44 | 1.42E+07 | 1.27E+05 | 42.05 | 2.82 | 332.20 | 1.07 | 40 | 35.03 | 28.47 | IN |
|  | 8784 | 3.00 |  | 226.6 | 2227.41 | 1.47E+07 | 1.30E+05 | 43.04 | 2.86 | 333.05 | 1.07 | 41 | 35.03 | 28.79 | IN |
|  | 8988 | 3.00 |  | 228.9 | 2250.38 | 1.52E+07 | 1.32E+05 | 44.04 | 2.90 | 333.79 | 1.06 | 42 | 35.03 | 29.11 | IN |
|  | 9191 | 3.00 |  | 231.2 | 2273.36 | 1.56E+07 | 1.35E+05 | 45.04 | 2.93 | 334.44 | 1.06 | 43 | 35.03 | 29.43 | IN |
|  | 9394 | 3.00 |  | 233.6 | 2296.33 | 1.61E+07 | 1.38E+05 | 46.03 | 2.97 | 334.99 | 1.06 | 44 | 35.03 | 29.75 | IN |
|  | 9598 | 3.00 |  | 235.9 | 2319.30 | 1.66E+07 | 1.41E+05 | 47.03 | 3.01 | 335.45 | 1.06 | 45 | 35.03 | 30.06 | IN |
|  | 9801 | 3.00 |  | 238.2 | 2342.28 | 1.71E+07 | 1.44E+05 | 48.03 | 3.05 | 335.83 | 1.06 | 46 | 35.03 | 30.38 | IN |
|  | 10005 | 3.00 |  | 240.5 | 2365.25 | 1.76E+07 | 1.46E+05 | 49.02 | 3.09 | 336.14 | 1.06 | 47 | 35.03 | 30.70 | IN |
|  | 10208 | 3.00 |  | 242.8 | 2388.22 | 1.81E+07 | 1.49E+05 | 50.02 | 3.13 | 336.37 | 1.06 | 48 | 35.03 | 31.02 | IN |
|  | 10411 | 3.00 |  | 245.1 | 2411.20 | 1.86E+07 | 1.52E+05 | 51.02 | 3.17 | 336.52 | 1.05 | 49 | 35.03 | 31.34 | IN |
|  | 10615 | 3.00 |  | 247.4 | 2434.17 | 1.92E+07 | 1.55E+05 | 52.01 | 3.21 | 336.62 | 1.05 | 50 | 35.03 | 31.66 | IN |
|  | 10668 | 3.00 |  | 248.0 | 2440.20 | 1.93E+07 | 1.56E+05 | 52.27 | 3.24 | 336.64 | 1.05 | 51 | 35.03 | 31.74 | IN |
| at GL | 8991 | 3.00 |  | 229.0 | 2250.75 | 1.52E+07 | 1.32E+05 | 41.61 | 2.92 | 315.32 | 1.13 | 52 | 35.03 | 29.12 | IN |
| Safety Factor |  |  | Limit | Min.SF: | 1.05 | Limiting SF: |  |  | 1.00 | Remarks: |  | PASSED |  |  |  |

| MAX LOAD |  |  |
| --- | --- | --- |
| d= | 500 | mm |
| L1= | 8491 | mm |
| Max Load FH= | 4900 | N |
| MOE E= | 21000000 | N/cm2 |
| Inertia Moment IB= | 1.52E+03 | cm4 |
| RC/RB n= | 0.55 |  |
| α1= | 0.5112 | Coefficie nt Table |
| β1= | 0.9091 |  |
| f CP= | 53.198269 | cm |
| △= | 5.92% |  |
| θ AP= | 0.1008572 | ° |

Deflection Calculation: Deflection Limit:

MAX Deflection:		53.1983 cm Limiting Deflection:			89.91 cm Remarks:	PASSED

| WORKING LOAD |  |  |
| --- | --- | --- |
| d= | 500 | mm |
| L1= | 8491 | mm |
| Max Load FH= | 4900 | N |
| MOE E= | 21000000 | N/cm2 |
| Inertia Moment IB= | 1.52E+03 | cm4 |
| RC/RB n= | 0.55 |  |
| α1= | 0.5112 | Coefficie nt Table |
| β1= | 0.9091 |  |
| f CP= | 53.198269 | cm |
| △= | 5.92% |  |
| θ AP= | 0.1008572 | ° |